ElectraHub CSMS

WebSocket Connection Management

High Level Design (HLD)

| Date | March 20, 2026 |
| --- | --- |
| Version | 1.0 |
| Platform | ElectraHub EV Charging Platform |
| Service | ocpp-service (Port 8082) |
| Scope | WebSocket lifecycle, distributed routing, event coordination |

# Table of Contents

1. Purpose and Scope

2. Business Context

3. Architecture Overview

4. Component Architecture

5. Connection Lifecycle

6. Distributed Coordination

7. Event Routing Architecture

8. Data Flow Patterns

9. Caching and State Management

10. Security Model

11. Resilience and Availability

12. Scalability Design

13. Observability

14. Technology Justification

15. Risks and Mitigations

# 1. Purpose and Scope

This document defines the high-level architecture for managing persistent WebSocket connections between the ElectraHub backend (ocpp-service) and EV charging stations. The OCPP protocol mandates long-lived WebSocket connections for bidirectional communication. This design addresses how the platform establishes, tracks, routes messages through, and recovers these connections across a distributed multi-node deployment.

## 1.1 Design Goals

Maintain persistent, bidirectional WebSocket connections with thousands of charging stations simultaneously.

Support horizontal scaling of ocpp-service instances behind a load balancer without breaking active connections.

Provide sub-second message routing from any backend service to the correct charger, regardless of which node holds the connection.

Ensure zero message loss during node failures through graceful degradation and automatic reconnection.

Maintain a centralized, consistent view of all active charger connections across the cluster.

## 1.2 Out of Scope

OCPP message payload parsing and business logic (covered in OCPP Service LLD).

Charging session lifecycle management (covered in Session Service design).

Load balancer configuration and TLS termination specifics.

# 2. Business Context

EV charging stations communicate with the backend exclusively through OCPP (Open Charge Point Protocol), which operates over persistent WebSocket connections. Unlike traditional REST APIs where each request is independent, OCPP requires the server to maintain long-lived connections so that it can both receive messages from chargers (status updates, meter values, authorization requests) and send commands to chargers (remote start/stop, configuration changes, firmware updates).

This bidirectional requirement creates a fundamental challenge in distributed systems: when a backend service needs to send a command to a specific charger, it must know which server node currently holds the WebSocket connection and route the message there. The connection management system is the critical infrastructure that makes this routing possible.

## 2.1 Scale Requirements

| Metric | Initial Target | Growth Target | Design Ceiling |
| --- | --- | --- | --- |
| Concurrent Connections | 1,000 | 10,000 | 50,000 |
| Messages per Second (inbound) | 500/s | 5,000/s | 25,000/s |
| Messages per Second (outbound) | 200/s | 2,000/s | 10,000/s |
| OCPP Service Nodes | 2 | 5 | 20 |
| Connection Duration | Hours to days | Days to weeks | Persistent |
| Reconnection Time (target) | < 30s | < 15s | < 5s |

# 3. Architecture Overview

The WebSocket connection management architecture is split into three layers: the Connection Layer (handles physical WebSocket sessions), the Coordination Layer (tracks connection-to-node mapping across the cluster), and the Routing Layer (delivers messages to the correct node and connection).

## 3.1 System Context Diagram

+-----------------+
                     |  Charging       |
                     |  Stations       |
                     +--------+--------+
                              |
                     WebSocket (OCPP)
                              |
                     +--------v--------+
                     |  Load Balancer  |
                     |  (Sticky/IP)    |
                     +---+----+----+---+
                         |    |    |
              +----------+    |    +----------+
              |               |               |
     +--------v---+  +--------v---+  +--------v---+
     | ocpp-svc   |  | ocpp-svc   |  | ocpp-svc   |
     | Node A     |  | Node B     |  | Node C     |
     +------+-----+  +------+-----+  +------+-----+
            |               |               |
     +------v---------------v---------------v------+
     |                  Redis Cluster               |
     |  (Connection Registry + Pub/Sub Routing)     |
     +-----------------------+---------------------+
                             |
     +-----------+-----------+-----------+
     |           |           |           |
  +--v---+  +----v--+  +----v---+  +----v----+
  |Station|  |Session|  |Billing |  | Admin   |
  |Mgmt   |  |Svc   |  |Svc     |  | Portal  |
  +-------+  +------+  +--------+  +---------+

## 3.2 Three-Layer Architecture

| Layer | Responsibility | Key Components | State Store |
| --- | --- | --- | --- |
| Connection Layer | WebSocket session lifecycle: handshake, heartbeat, close | OcppWebSocketHandler, ConnectionManager | Local ConcurrentHashMap |
| Coordination Layer | Cluster-wide connection registry and node discovery | Redis Connection Registry, HeartbeatMonitorService | Redis (key-value + TTL) |
| Routing Layer | Cross-node message delivery for outbound commands | Redis Pub/Sub, MessageBridge, RemoteCommandService | Redis Pub/Sub channels |

# 4. Component Architecture

## 4.1 ocpp-service Internal Components

+--------------------------------------------------------------+
|                      ocpp-service Node                       |
+--------------------------------------------------------------+
|                                                              |
|  +-------------------+     +------------------------+       |
|  | WebSocket Handler |---->| Connection Manager     |       |
|  | (OCPP Handshake)  |     | - Local Session Map    |       |
|  +--------+----------+     | - Redis Registry Sync  |       |
|           |                +----------+-------------+       |
|           v                           |                     |
|  +-------------------+     +----------v-------------+       |
|  | Message Router    |     | Redis Connection       |       |
|  | (CALL dispatch)   |     | Registry               |       |
|  +--------+----------+     | ocpp:conn:{cpId}=nodeId|       |
|           |                +------------------------+       |
|           v                                                 |
|  +-------------------+     +------------------------+       |
|  | Message Handlers  |     | Redis Pub/Sub Bridge   |       |
|  | (Boot/Auth/Meter) |     | Sub: ocpp:cmd:{nodeId} |       |
|  +-------------------+     | Pub: ocpp:cmd:{target} |       |
|                            +------------------------+       |
|  +-------------------+     +------------------------+       |
|  | Remote Command    |---->| Pending Response Map   |       |
|  | Service           |     | ConcurrentHashMap      |       |
|  +-------------------+     | <msgId, Future>        |       |
|                            +------------------------+       |
|  +-------------------+     +------------------------+       |
|  | Heartbeat Monitor |---->| Stale Connection       |       |
|  | (Scheduled)       |     | Reaper                 |       |
|  +-------------------+     +------------------------+       |
+--------------------------------------------------------------+

## 4.2 Component Responsibilities

| Component | Responsibility | Concurrency Model |
| --- | --- | --- |
| OcppWebSocketHandler | WebSocket lifecycle (connect, message, disconnect, error). Extracts chargePointId from URI, delegates to ConnectionManager and MessageRouter. | One thread per WebSocket session (Spring default) |
| ConnectionManager | Dual-write connection registry: local ConcurrentHashMap for O(1) session lookup, Redis for cluster-wide awareness. Generates unique nodeId per instance. | Thread-safe via ConcurrentHashMap + atomic Redis ops |
| OcppMessageRouter | Dispatches inbound CALL messages to registered handlers. Routes CALL_RESULT/CALL_ERROR to RemoteCommandService for future resolution. | Stateless, handler lookup is read-only after init |
| RemoteCommandService | Sends outbound CALL messages to chargers. Manages pending response futures with configurable timeout. | ConcurrentHashMap for pending futures, CompletableFuture for async |
| Redis Pub/Sub Bridge | Subscribes to node-specific channel. Publishes outbound commands to target node channel. Enables cross-node message delivery. | Dedicated subscriber thread per node |
| HeartbeatMonitorService | Scheduled scan for stale connections. Reaps connections exceeding heartbeat timeout. | Single scheduled thread, DB + ConnectionManager coordination |
| OcppMessageLogService | Persists all OCPP messages for audit trail and debugging. | Async persistence, non-blocking to message flow |

# 5. Connection Lifecycle

## 5.1 Connection States

Each charging station connection follows a defined state machine from initial handshake through active operation to eventual disconnection.

| State | Description | Trigger | Next States |
| --- | --- | --- | --- |
| CONNECTING | WebSocket handshake in progress | Client initiates WS connection | CONNECTED, REJECTED |
| CONNECTED | WS session established, awaiting BootNotification | Handshake complete | AWAITING_BOOT, DISCONNECTED |
| AWAITING_BOOT | Waiting for charger to send BootNotification | Connection established | REGISTERED, DISCONNECTED |
| REGISTERED | BootNotification accepted, charger operational | BootNotification Accepted | ONLINE, DISCONNECTED |
| ONLINE | Active operation, heartbeats flowing | First heartbeat after registration | OFFLINE, DISCONNECTED |
| OFFLINE | Heartbeat timeout exceeded | HeartbeatMonitor detects stale | ONLINE, DISCONNECTED |
| DISCONNECTED | WebSocket session closed | Transport error, explicit close, reap | CONNECTING (reconnect) |
| REJECTED | Connection denied (auth failure, blacklist) | Security check failed | Terminal |

## 5.2 Connection Establishment Flow

When a charging station connects, the ocpp-service performs the following sequence: validate the WebSocket upgrade request (check chargePointId format, OCPP subprotocol negotiation), register the session locally in the ConnectionManager ConcurrentHashMap, publish the connection to Redis with the node identifier, persist an OcppConnection entity to PostgreSQL for audit, and begin accepting OCPP messages. The charger is expected to send a BootNotification within a configurable timeout; failure to do so transitions the connection to a reapable state.

## 5.3 Graceful Disconnection

On disconnection (whether initiated by the charger, triggered by error, or reaped by the heartbeat monitor), the system removes the session from the local map, deletes the Redis connection key, updates the OcppConnection entity with disconnectedAt timestamp, and cancels any pending outbound command futures for that charger.

# 6. Distributed Coordination

## 6.1 Redis Connection Registry

The connection registry uses Redis as a lightweight, fast, distributed key-value store to map each connected charger to the node currently holding its WebSocket session. This is the single source of truth for cross-node connection resolution.

Key Design:

Key:    ocpp:conn:{chargePointId}
Value:  {nodeId}:{sessionId}:{connectedAt}
TTL:    180 seconds (auto-expire on missed refresh)

Example:
  ocpp:conn:CP-001 = node-a:ws-sess-abc:1711000000
  ocpp:conn:CP-002 = node-b:ws-sess-def:1711000100

Justification for Redis:

Sub-millisecond read latency for connection lookups on the message routing path.

Native TTL support eliminates stale entries from crashed nodes without manual cleanup.

Pub/Sub capability reused for cross-node message routing (single infrastructure component).

Atomic operations (SET NX, DEL) prevent race conditions during connect/disconnect.

Already in the technology stack for session caching and other platform needs.

## 6.2 Node Identity and Health

Each ocpp-service instance generates a unique node identifier at startup (from HOSTNAME environment variable or a generated UUID). The node registers itself in Redis with a heartbeat TTL. A scheduled task refreshes the node heartbeat every 30 seconds. If a node crashes, its heartbeat expires and the HeartbeatMonitorService on surviving nodes detects orphaned connections.

Node Registry:
  Key:    ocpp:node:{nodeId}
  Value:  {hostname}:{port}:{startedAt}:{connectionCount}
  TTL:    60 seconds (refreshed every 30s)

## 6.3 Connection Handoff (Rolling Deployment)

During rolling deployments, a draining node stops accepting new connections but maintains existing ones until a configurable grace period expires. Chargers that lose connections during shutdown automatically reconnect to another node via the load balancer. The Redis registry naturally transitions as old entries expire and new entries are created on the receiving node.

# 7. Event Routing Architecture

## 7.1 Inbound Message Routing (Charger to Backend)

Inbound messages arrive directly on the WebSocket connection and are processed locally by the node holding the connection. No cross-node routing is needed for inbound messages.

Charger ---[WebSocket]---> ocpp-service Node
                            |
                     OcppWebSocketHandler
                            |
                     OcppMessageRouter
                            |
               +------------+------------+
               |            |            |
         CALL handler  CALL_RESULT  CALL_ERROR
         (business     (resolve     (reject
          logic)        future)      future)

## 7.2 Outbound Message Routing (Backend to Charger)

Outbound messages (remote commands) require cross-node routing because the requesting service may call any ocpp-service node via the load balancer, but the target charger may be connected to a different node.

Backend Service ---[REST]---> ocpp-service Node X (any node)
                                    |
                           ConnectionManager.getSession(cpId)
                                    |
                        +-----------+-----------+
                        |                       |
                  LOCAL (found)           REMOTE (not found)
                        |                       |
                  Send directly         Redis lookup: nodeId
                  via WebSocket                  |
                                       Pub/Sub -> ocpp:cmd:{nodeId}
                                                |
                                       Target Node receives
                                       and sends via WebSocket

## 7.3 Redis Pub/Sub Channel Design

Each ocpp-service node subscribes to its own command channel. When a node needs to send a command to a charger on another node, it publishes the command to that node's channel.

Channel Pattern:  ocpp:cmd:{nodeId}

Message Envelope:
{
  "messageId": "uuid",
  "chargePointId": "CP-001",
  "action": "RemoteStartTransaction",
  "payload": { ... },
  "originNodeId": "node-x",
  "timestamp": 1711000000
}

Response Channel: ocpp:resp:{originNodeId}
(for routing CALL_RESULT back to requesting node)

# 8. Data Flow Patterns

## 8.1 Inbound: StatusNotification

When a charger reports a connector status change, the message flows directly from the WebSocket handler through the message router to the StatusNotificationHandler, which calls station-management-service via REST to persist the status change.

## 8.2 Outbound: RemoteStartTransaction

A driver app initiates charging via the REST API. The request reaches any ocpp-service node. That node checks its local ConnectionManager for the charger. If local, it sends directly. If remote, it looks up the target node in Redis and publishes the command via Pub/Sub. The target node receives the command, sends it over the WebSocket, and publishes the charger's response back to the origin node.

## 8.3 Bidirectional: Authorize + StartTransaction

The most complex flow involves the charger sending an Authorize request (inbound), the backend validating it with session-service (REST), responding to the charger, then the charger sending StartTransaction (inbound), and finally the backend creating a session in session-service. This is fully local to the node holding the connection.

# 9. Caching and State Management

| State | Storage | Access Pattern | Justification |
| --- | --- | --- | --- |
| Active WS Sessions | Local ConcurrentHashMap | O(1) lookup by chargePointId | Sub-microsecond latency for message dispatch. Cannot serialize WebSocket sessions. |
| Connection-to-Node Map | Redis (key-value + TTL) | O(1) lookup, auto-expire | Distributed consensus on which node owns which connection. TTL handles crash recovery. |
| Node Registry | Redis (key-value + TTL) | Periodic scan for health | Cluster membership and health monitoring. TTL-based liveness detection. |
| Pending Command Futures | Local ConcurrentHashMap | O(1) insert/resolve by messageId | CompletableFuture cannot be serialized. Timeout-based cleanup for orphaned entries. |
| Connection History | PostgreSQL (OcppConnection) | Append + query by chargePointId | Durable audit trail. Time-range queries for diagnostics. |
| Message Log | PostgreSQL (OcppMessageLog) | Append + query by time/chargePoint | Compliance and debugging. Async write to avoid blocking message path. |

Data Structure Justification:

ConcurrentHashMap is chosen for local state because WebSocket session objects cannot be serialized or shared across JVM instances. The combination of local HashMap (for performance) plus Redis (for coordination) follows the established pattern of cache-aside with a distributed coordination layer. Redis TTL eliminates the need for distributed garbage collection of stale entries from crashed nodes.

# 10. Security Model

## 10.1 Connection Authentication

Charging stations authenticate during the WebSocket handshake using OCPP Security Profiles. The system supports three profiles with increasing security levels.

| Profile | Method | Description |
| --- | --- | --- |
| Profile 1 | Basic Authentication | Username/password in WS handshake headers. Credentials validated against station-management-service. |
| Profile 2 | TLS with Server Certificate | Server presents TLS certificate. Station validates server identity. Basic auth may still be used. |
| Profile 3 | TLS with Client Certificate | Mutual TLS (mTLS). Both station and server present certificates. Strongest authentication. |

## 10.2 Internal Communication Security

Inter-service REST calls use X-Internal-Api-Key header validation.

Redis connections use AUTH password and optionally TLS in production.

Pub/Sub messages include origin node verification to prevent spoofing.

WebSocket endpoints are not exposed through the API Gateway; chargers connect directly to the ocpp-service load balancer.

# 11. Resilience and Availability

## 11.1 Failure Scenarios and Recovery

| Failure | Detection | Recovery | Impact |
| --- | --- | --- | --- |
| Node crash | Redis TTL expiry (60s) | Chargers auto-reconnect to surviving nodes via LB | Temporary loss of connections on crashed node |
| Redis unavailable | Connection timeout | Fall back to local-only mode; disable cross-node routing | Cross-node commands fail; local connections work |
| Network partition | Heartbeat failures | Split-brain prevention via Redis node heartbeats | Possible duplicate connection registrations |
| Charger disconnect | WS close event / heartbeat timeout | Clean up local + Redis state; charger reconnects | Pending commands fail with timeout |
| High memory (OOM) | JVM metrics / container limits | Connection admission control (max connections per node) | Reject new connections; existing ones maintained |
| Message storm | Rate limiting counters | Per-charger rate limiting + backpressure | Excess messages dropped with warning |

## 11.2 Graceful Shutdown

On SIGTERM, the node enters draining mode: stops accepting new connections, sends CloseFrame to all connected chargers with a retry-after hint, waits for pending commands to complete (up to 30s), then shuts down. Chargers will automatically reconnect to another node.

# 12. Scalability Design

## 12.1 Horizontal Scaling Model

The system scales horizontally by adding more ocpp-service nodes. The load balancer distributes new WebSocket connections using IP-hash or sticky sessions. Each node independently manages its connections while Redis provides cluster coordination.

| Scale Factor | Strategy | Bottleneck | Mitigation |
| --- | --- | --- | --- |
| Connections | Add nodes; 5,000-10,000 connections per node | File descriptors, memory | Tune ulimits, use off-heap buffers |
| Inbound messages | Local processing, no cross-node hop | CPU (message parsing) | Handler thread pool sizing |
| Outbound commands | Redis Pub/Sub fan-out | Redis throughput | Redis Cluster with sharding |
| Connection registry | Redis key-per-connection | Redis memory | Key expiry, efficient encoding |

## 12.2 Connection Limits

Each node enforces a configurable maximum connection count (default: 10,000). When the limit is reached, new connections receive a 503 Service Unavailable response. Kubernetes HPA can auto-scale based on the connection count metric.

# 13. Observability

## 13.1 Metrics

| Metric | Type | Labels | Purpose |
| --- | --- | --- | --- |
| ocpp_connections_active | Gauge | node, protocol | Current active connections per node |
| ocpp_connections_total | Counter | node, result | Total connection attempts (accepted/rejected) |
| ocpp_messages_inbound_total | Counter | node, action | Inbound messages by OCPP action |
| ocpp_messages_outbound_total | Counter | node, action, result | Outbound commands by action and result |
| ocpp_message_latency_seconds | Histogram | action, direction | Message processing latency |
| ocpp_routing_hops | Counter | type | Local vs cross-node routing counts |
| ocpp_heartbeat_age_seconds | Gauge | charge_point_id | Time since last heartbeat |
| ocpp_redis_operations_total | Counter | operation, result | Redis registry operations |

## 13.2 Structured Logging

All connection lifecycle events are logged in structured JSON format with correlation fields: chargePointId, nodeId, sessionId, action, messageId. This enables efficient log queries for diagnosing connection and routing issues.

## 13.3 Health Endpoints

/actuator/health: Standard Spring Boot health including Redis and DB connectivity.

/actuator/health/websocket: Custom health indicator showing connection count and node status.

/api/v1/ocpp/connections/count: Active connection count for load balancer health checks.

# 14. Technology Justification

| Technology | Purpose | Alternatives Considered | Rationale |
| --- | --- | --- | --- |
| Spring WebSocket | WebSocket server | Netty, Undertow, Vert.x | Native Spring Boot integration, simpler operational model, sufficient for target scale. |
| ConcurrentHashMap | Local session store | Caffeine cache, Guava cache | Sessions cannot be evicted (active connections). No TTL needed locally. Lowest overhead. |
| Redis (Key-Value) | Connection registry | Consul, etcd, ZooKeeper | Already in stack. Sub-ms reads. Native TTL. Pub/Sub for routing in same system. |
| Redis Pub/Sub | Cross-node routing | RabbitMQ, Kafka, gRPC streaming | Lowest latency for fire-and-forget commands. No persistence needed (commands are ephemeral). Same Redis instance as registry. |
| PostgreSQL | Connection audit log | Elasticsearch, ClickHouse | Already in stack. ACID for audit compliance. Sufficient write throughput for connection events. |
| CompletableFuture | Async command response | Project Reactor, RxJava | Simpler API for request-response pattern. No need for full reactive stack. |

# 15. Risks and Mitigations

| Risk | Impact | Probability | Mitigation |
| --- | --- | --- | --- |
| Redis single point of failure | All cross-node routing fails | Low | Redis Sentinel or Cluster for HA. Local-only fallback mode. |
| Split-brain: charger connected to two nodes | Duplicate message processing | Low | Redis SETNX for atomic connection claim. Last-writer-wins with version check. |
| WebSocket memory pressure | Node OOM, connection drops | Medium | Per-node connection limits. Message size limits. Memory-aware admission control. |
| Pub/Sub message loss | Outbound commands silently dropped | Low | Persistent pending request table. Timeout-based retry. Dead letter logging. |
| Thundering herd on node restart | All chargers reconnect simultaneously | Medium | Randomized reconnect backoff in charger firmware. Connection rate limiting on server. |
| Long-running Pub/Sub subscriber blocking | Message delivery delays | Low | Dedicated subscriber thread pool. Message queue buffering. |